Learning Roadmap

We'll treat this like a mini industry project.

Phase 1 — Understanding CMOS Inverter

Before opening Virtuoso, understand:

- NMOS operation

- PMOS operation

- Threshold voltage

- Cutoff

- Linear region

- Saturation

- CMOS switching

Goal:

Understand why CMOS consumes almost zero static power.

Phase 2 — Schematic Design

In Virtuoso:

Create

Library
   ↓
Cell
   ↓
View (schematic)

Then place

PMOS
NMOS
VDD
GND
Input
Output

Result

VDD
       |
      PMOS
       |
OUT-----------
       |
      NMOS
       |
      GND

Input connected to both gates

Phase 3 — Symbol Creation

From the schematic create a symbol.

Design
→ Create Cellview
→ From Cellview

You'll obtain a reusable inverter block.

Phase 4 — Testbench

Create another schematic.

Library
   ↓
tb_inverter

Add

- inverter symbol

- VDD

- Pulse source

- Load capacitor

Pulse ---> Inverter ---> Output

           |
          VDD

Phase 5 — DC Analysis

Use ADE Explorer.

Run

Analysis
→ dc

Sweep

Vin
0 → VDD

Observe

Vout vs Vin

This is the Voltage Transfer Characteristic (VTC).

You'll learn:

- VOH

- VOL

- VIH

- VIL

- Switching threshold

- Noise margins

These are fundamental concepts in digital logic and analog device behavior.

Phase 6 — Transient Analysis

Apply a pulse input.

0
↓

VDD
↓

0
↓

VDD

Measure:

- Rise time

- Fall time

- Propagation delay

- Average power

This is how real digital circuits are evaluated.

Phase 7 — Parametric Analysis

Change transistor sizes.

Example:

NMOS W = 1 µm

PMOS W = 2 µm

Then try

PMOS = 3 µm

PMOS = 4 µm

PMOS = 5 µm

Observe:

- Delay

- Power

- Switching point

You'll understand why PMOS is often made about 2–3× wider than NMOS to balance drive strength.

Phase 8 — Layout

Create the physical layout.

You'll learn:

- Diffusion

- Poly

- Metal1

- Metal2

- Contacts

- Wells

This is where IC design becomes tangible.

Phase 9 — DRC

Run

Verify
→ DRC

Fix every design-rule violation.

Phase 10 — LVS

Run

Verify
→ LVS

Ensure

Layout == Schematic

If it passes, your layout correctly implements the intended circuit.

Phase 11 — Parasitic Extraction (PEX)

Extract

- Wire resistance

- Wire capacitance

- Coupling capacitance

These parasitics affect real-chip performance.

Phase 12 — Post-Layout Simulation

Simulate the extracted netlist.

Compare:

| Metric | Pre-layout | Post-layout |
| --- | --- | --- |
| Delay | ✓ | ✓ |
| Power | ✓ | ✓ |
| Rise time | ✓ | ✓ |
| Fall time | ✓ | ✓ |

You'll see how layout-induced parasitics impact circuit behavior.

CADENCE cannot work on WINDOWS. It needs some open source OS

1. File -> New -> Library -> (Name), Attach to existy technology -> gpdk90

2. File -> New -> Cell Vie  w-> (Name)